CALCULATION of the characteristics of ION DRIFT IN gas BY I_DRIFT CODE

R.I. Golyatina, S.A. Maiorov

A.M. Prokhorov General Physics Institute, Moscow, Russia, mayorov_sa@mail.ru

The description of the program iDrift, which calculates the characteristics of the ion drift in a gas at a constant and uniform electric field. The gas may be your own (the same kind that the ion), and the improper mixture of gases of different concentrations. We consider a swarm of ions, each of which starts from the initial point and accelerated by a constant and uniform electric field. At some point of time, there is a ion-atom collision, the probability of such an event is played out from the known collision cross sections of different types.

Considered a collision with resonance charge transfer, polarization interaction and repulsion due to the interaction of the electron shells. Model of ion-atom collisions, implemented by Monte Carlo, can properly take into account the energy balance of the ions as they drift [1].

This formulation of the problem allows us to calculate all the kinetic transport coefficients are needed to simulate a spatially inhomogeneous plasma and non-stationary processes in it:

- Coefficient of ion mobility and diffusion along and across the field,

- Distribution function of the velocity in the direction along and across the field,

- Angular and energy distribution of ions bombarding the surface.

The last two characteristics determine, for example, the efficiency and the quality of the ion etching of semiconductor wafers in a plasma chemical reactors.

This program is used mainly for modeling of transport processes in dusty plasma, where the ion-atom collisions, despite their rarity, may have a decisive influence on the characteristics of the dust subsystem in plasma [2, 3].

In addition to the experiments in normal conditions considered as experiments at cryogenic temperatures, the gas [4], and mixtures of the noble gases [5, 6]. Completed program iDrift calculations [7] revealed a deep analogy between the discharge and the discharge cryogenically in a mixture of light and heavy gas, led to significant findings for the theory of the applicability of the widely used BGK approximation [8].

The study was supported by The Ministry of education and science of Russia, project 8392.

References

- Maiorov S.A Physics of Plasmas, 2009, Т. 35, №9, с. 869-880.

- Maiorov S.A., Physics of Plasmas, 2005, Т. 31, №8, с.749.

- Maiorov S.A., Physics of Plasmas, 2006, Т. 32, №9, с. 802-815.

- Antipov S.N., Asinovski A.I., Kirillin А.V., Maiorov S.A., Markovets V.V., Petrov О.F., Fortov V.Е., JETP, 2008, 133, issue 4, 948.

- Maiorov S.A., Ramazanov T.S., Dzhumagulova K.N., Jumabekov A.N.  and Dosbolaev A.N., Physics of Plasmas, 2008, 15, 093701.

- Antipov S.N., Vasilev М.М., Maiorov S.A., Petrov О.F., Fortov V.Е., JETP, 2011, 139, issue 3, 554.

- Golyatina R.I., Maiorov S.A., Brief reports on physics LPI, 2012,  № 9, 30.

- Maiorov S.A., Citovich V.N., Brief reports on physics LPI, 2012,  № 3, 14.