DOI: 10.34854/ICPAF.2020.47.1.076

TIME- AND SPACE-RESOLVED X-RAY EMISSION FROM HYFRID X-PINCH *)

1Pikuz S., 1Shelkovenko T., 1Tilikin I., 2Elshafiey A., 2Hammer D.

1P.N. Lebedev Physical Institute, Moscow, Russia, office@email.ru
2Cornell University, Ithaca, USA, csv8@cornell.edu

The report presents the results of x-ray studies of hybrid X-pinch at the Cornell University XP facility (current pulse amplitude of 400 kA with a front duration of 100 ns) [1,2]. The goal of this work is to determine the main mechanisms for the formation of a pinch hotspot, including radiation collapse. At the final stage of matter compression in a fast pinch, all processes occur in less than 1 nanosecond [2]. In this case, the plasma parameters change by several orders of magnitude, reaching "stellar" values. Information about processes in plasma can be obtained by examining its x-ray dynamics with a time resolution of the order of several picoseconds. This raises the problem of synchronizing the recording equipment with the process under study, which is stochastic in nature. The choice of a hybrid X-pinch was determined by its significantly better reproducibility compared to the standard X-pinch. A measurement system based on Kentech x-ray chronogaphs, diamond sensors, broadband scopes and spectrographs with spherically bent crystals was created for synchronous study of the temporal, spatial, and spectral characteristics of pinch radiation [3]. Data on the dynamics and parameters of the plasma of hybrid X-pinches with loads from different materials were obtained.

This research is sponsored by DE-SC0018088, as well as by NNSA/SSAP under DOE Cooperative Agreement DE-NA0003764

References

T.A. Shelkovenko et al., Phys. Plas., 17, 112707, 2010.

T.A. Shelkovenko et al., Plas. Phys. Rep. 38, 359-381, 2012.

S.A. Pikuz, T.A. Shelkovenko, D.A. Hammer, Plas. Phys. Rep. 41, 445-491, 2015.

S.A. Pikuz, T.A. Shelkovenko, D.A. Hammer, Plas. Phys. Rep. 41, 291-342, 2015.

*)  abstracts of this report in Russian