# STATIONARY AND NON-STATIONARY DISCHARGE AT ATMOSPHERIC PRESSURE IN PIN-PLATE SYSTEM AND ITS APPLICATION FOR NONEQUILIBRIUM PLASMACHEMISTRY

V.I. Golota, V.I. Karas', *V.P. Mal'khanov, **I.F. Potapenko, O.N. Shulika

National Science Center "Kharkov Institute of Physics & Technology", Kharkov, Ukraine

* TurboDEn, Moscow, Russia

** Keldysh Institute of Applied Mathematics RAS, Moscow, Russia

In this report we considered the following problems:

- Theoretical studies of a stationary and non-stationary discharge at atmospheric pressure in "pin-plate" system;

- Numerical simulation of physical processes in "pin-plate" system;

- Experimental investigations of atmospheric pressure discharge;

- Development of ecological pure technologies for:

  - ozone generation;

  - fume and exhausted gases cleaning;

  - natural gas processing.

Theoretical studies and numerical simulation for definite gas mixture include the following steps:

- Solving Poisson equation for nine components (electrons, positive ions of two sorts, negative ions of two sorts, charged molecules and clusters) and calculation of reduced electric field distribution;

- Solving Boltzmann equation for electron distribution function fε,E/N,σije with taking into account of elastic and non-elastic processes (ionization, recombination, attachment, detachment, excitation of oscillating, rotating and electron levels by electrons and processes of II type);

- Calculation (using electron distribution function) of ionization, attachment, detachment, recombination and excitation coefficients; and also deriving both drift velocities and diffusion coefficients for ions, electrons, charged molecules and clusters;

- Solving a continuity equations set;

- Solving an equation for electric circuit (Kirchoff equation);

- Iteration of steps 1-5;

- Calculating the chemical reactions in the gas mixture;

- Improvement of the gas mixture content;

- Iteration of steps 1-8.

For calculations of a spatially non-uniform gas discharge we applied a one-dimensional approach. As a result the distributions of reduced electric field, charged particles concentrations and concentrations of both output products and active radicals were obtained.

In this report we presented a comparison between the theoretical and experimental results for positive and negative polarity on needle electrode. We also discussed an influence of electrode polarity on following phenomena:

- transformation of steady-state discharge into non-steady-state one;

- variety of voltage-current characteristics;

- increasing of efficiency factor for positive polarity on needle electrode.

In particular, we used 2-D approximation for consideration of non-steady glow discharge.

This work was supported by Science and Technology Center in Ukraine (grant # 1069).

e-mail:

V.I.Golota - Ukraine, Kharkov, NSC KIPT, golota@kipt.kharkov.ua

V.I.Karas' - Ukraine, Kharkov, NSC KIPT, karas@kipt.kharkov.ua

I.F.Potapenko - Russia, Keldysh IAM RAS, irina@KELDYSH.ru

O.N.Shulika - Ukraine, Kharkov, NSC KIPT, unicorn@kipt.kharkov.ua