Measurements of velocity distribution function of Ar II ions in PNX-U maschine plasma with laser spectroscopy technique

I.V. Moskalenko, N.A. Molodtsov, A.A. Skovoroda, D.A. Shcheglov

Nuclear Fusion Institute RRC “Kurchatov Institute”, Moscow, Russia,
e-mail: ivm@nfi.kiae.ru

Ar II ion radial velocity distribution function (VDF) was measured in multicusp trap PNX-U [1,2] under ECR heating conditions with unput power up to ~ 50 kW. Temporal variation of Doppler “effective temperature” has been measured for various spatial points. The beam of narrow band (L  3.6 pm) laser radiation was directed along diameter of plasma column; the range -ro/2 < r < ro/2 (ro=30 см). Ar II fluorescence scheme which was used in the measurements is shown in figure. The target state was metastable state; the VDF was measured by scanning across Doppler-broadened 611.5 nm absorption line and by detection fluorescence light at 461-nm line. Making use FWHM Doppler width corrected for instrumental broadening “effective temperatures” Teff,i were measured (Teff,i are in the range 4-25 eV).The analysis of the experimental results leads to conclusion on “anomalous” heating of argon ions. The concept of positive potential barrier existing during microwave power injection [1] was used to explain the phenomenon.

The measurements described above were simultaneously the proof-of-principal testing of laser spectroscopy method and prototype of the tunable laser transmitter being developed for ITER-FEAT divertor plasmas diagnosis by LIF-technique [3]. During the studies, the ITER requirements on spatial  and temporal resolution and other measurements specification were taken into account.

References

- Zhil’tsov V.A., Klimenko E.Yu., Kosarev P.M., e.a., Nuclear Fusion, 2000, Vol. 40, P. 509.

- Kulygin V.M., Dlougach E.D., Gorbunov E.P., e.a., Nuclear Fusion, 2001, Vol. 41, P. 355.

- Donne A.J.H., e.a., Proc. 10th LAPD Symposium, Furuoka, Japan, 2001, P. 389.

Authors

Moskalenko I.V., Russia, RF, NFI RRC “Kurchatov institute”, e-mail: ivm@nfi.kiae.ru

Molodtsov N.A., Russia, RF, NFI RRC “Kurchatov institute”

Skovoroda A.A., Russia, RF, NFI RRC “Kurchatov institute”, e-mail: skovorod@nfi.kiae.ru

Shcheglov D.A., Russia, RF, NFI RRC “Kurchatov institute”, e-mail: shchegl@nfi.kiae.ru