STUDY OF ELECTRON HEAT PULSE PROPAGATION WITH TWO FREQUENCIES SET OF GYROTRONS ON T-10 TOKAMAK

Neudatchin S.V., Kislov A.Ya., Krupin V.A., Lysenko S.E., Pavlov Yu.D., Razumova K.A., Syshkov A.V., Borshagovskii A.A., Chistyakov V.V., Ilin V.I., Poznyak V.I., Roi I.N.

Nuclear Fusion Institute, RRC "Kurchatov Institute", Moscow, e-mail: neudatchin@nfi.kiae.ru

The 130 GHz gyrotron was recently installed on T-10 in addition to the set of 4 gyrotrons with 140 GHz frequencies.  Sawteeth-free plasma was created either by high-field-side off-axis ECRH (140 GHz) or by low-field-side off-axis ECRH (130 GHz) at r/a  +/- (0.45). Outward heat pulse propagation (HPP) with dynamic electron heat diffusivity eHP 0.20.3 m2/s (at 0.2< r/a < 0.37 and R/LTe =RTe/ Te up to 23) was created by switching on the on-axis ECRH (130 or 140 GHz) imposed on the background, which was created by off-axis ECRH. Our pioneer result is fully confirmed [1]. Inward cold pulse propagation (CPP) was created by turning-off the off-axis ECRH. The central electron temperature Te(0) responds with significant ~20ms delay. Slow inward CPP with R/LTe rises up to 17 on the cold wave front, can be described well by eHP  0.1 m2/s at r/a <0.3 in a time interval limited by the appearance of the first sawteeth oscillation. Off-axis ECRH forms the region of the improved transport in low-shear zone with q slightly above 1. Later, q reduces due to the current redistribution during CPP (HPP). The region of the improved transport is able to survive under R/LTe up to 23 (2.3 Ohmic value of R/LTe). Ohmic value of R/LTe lies above the “critical” one because enhanced HPP (induced by sawteeth or on-axis ECRH with eHP  1-2 m2/s) is observed at all tokamaks at Ohmic background. Somehow, off-axis ECRH increases the “critical” value of R/LTe over 2.5 times. The last T-10 experiments show the exact importance of the vicinity to q=1 for slow CPP with eHP  0.1 m2/s because under stronger current redistribution (e.g. due to increase of non-central ECRH power or by further shift of the resonance position away from centre) we observe the increase of  eHP  values.The analysis of HPP (CPP) at both sides of plasma column simultaneously shows light asymmetry of eHP values and confirms the validity of the results. Part of the results described above was reported in [2].

- Bagdasarov A. A., Vasin N. L., Neudatchin S. V., Savrukhin P. V. 1991 (Proc. 15th Int. IAEA Conf., Washington, 1990), Vol. 1 (IAEA: Vienna, 1991) 253

- Neudatchin S V et al 2002 Strong Microwaves in Plasmas (Int. Workshop in Nyznii Novgorod, Russia) rep. H-19, submitted to Nucl. Fus.