IR IMAGING INTERFEROMETER FOR OBSERVATION OF DENSITY AND FLUCTUATIONS PROFILES ON LHD

K. Tanaka, L. N. Vyacheslavov*, T. Akiyama**, A. Sanin, K. Kawahata, T. Tokuzawa, Y. Ito, S. Tsuji-Iio **, S. Okajima***

National Institute for Fusion Science, Toki, 509-5952,Japan

*Budker Institute of Nuclear Physics, 630090, Novosibirsk, RUSSIA,
email: vyachesl@inp.nsk.su

**Tokyo Institute of Technology, Meguro-ku, Tokyo 152-8550, Japan

***Chubu University, Kasugai 487-8501, Japan

The basic elements of the interferometer are three vertical slab probe beams those cover almost entire plasma cross-section. Phase modulations of the beam wave fronts produced by plasma density inhomogeneities are imaged into multichannel linear detector arrays. Two operational modes are employed: two color (1=10.6 m, 2= 1.06 m) heterodyne interferometer, which is used mostly for density profile observation, and phase contrast imaging (1=10.6 m) designed for study of plasma density fluctuation. The entire optical system including lasers is mounted on the massive vibration protected frame routinely used by 13ch FIR  (3=119 m) interferometer. Besides transversal spatial resolution different methods are destined for spatial resolution along the viewing line: magnetic shear techniques, defocused imaging and crossed beams correlation. The imaging interferometer is installed on the large helical device (LHD) and first results obtained in plasma experiments are reported.