STIFFNESS AND RIGIDITY OF TEMPERATURE PROFILE IN TOKAMAKS

Yu.N. Dnestrovskij, A.Yu. Dnestrovskij, S.E. Lysenko

Russian Research Centre ‘Kurchatov Institute’, Nuclear Fusion Institute, Moscow, Russia,
e-mail: dnyn@nfi.kiae.ru

Conceptions of stiffness and rigidity of temperature profile in a tokamak are introduced and formulated. The stiffness means the profile consistency under variations of the plasma density, boundary temperature and heating power; rigidity means a sharp dependence of the central temperature on the boundary one. Analytical estimations and numerical calculations, which use a model with critical gradients, show that at low boundary temperature (L-mode) the profiles, as a rule, are stiff, but not rigid. In the Н-mode, when the temperature pedestal is rather high, the range of parameters, where the profiles are stiff and rigid, exists. However, if we strongly increase the pedestal temperature, maintaining a rather flat profile of deposited power, the temperature profile can lose both rigidity and stiffness. Such a behavior of the electron temperature profiles has been observed in some high-density Н-mode shots on DIII-D and JET tokamaks. Rigidity and stiffness of profiles in MAST and T-10 are analyzed as well.