PREPARATION OF EXPERIMENT ON PLASMA HEATING BY NEUTRAL BEAM INJECTION ON TUMAN-3M TOKAMAK.

Askinazi L.G., *Barsukov A.G., Zhubr N.A., Kornev V.A., *Kuznetsov V.V., Lebedev S.V., *Panasenkov A.A., Razumenko D.V., *Tilinin G.N.

Ioffe Institute RAS, St.-Petersburg, RF, e-mail: Vladimir.Kornev@mail.ioffe.ru

*RRC “ Kurchatov Institute”, Moscow, RF, e-mai: tilinin@nfi.kiae.ru

Preparation of neutral beam injection experiment on TUMAN-3M tokamak is currently in the final stage in Ioffe Physico-Technical Institute [1], [2]. Auxiliary plasma heating by neutral beam injection (NBI) is expected to increase  (plasma pressure to magnetic field pressure ratio) substantially. Heating impact on plasma parameters has been estimated by energy balance modeling with the use of Merezhkin-Mukhovatov transport coefficients [3]. Our estimations show that this injector [4] (Pin0.6МW) will allow an increase in plasma ion temperature from 0.15 – 0.2 up to 0.6 – 0.8 keV, resulting in   2%, which is close to the ideal MHD modes stability limit for TUMAN-3M tokamak.

The paper describes TUMAN-3M NBI setup, employed PC-based acquisition and control system. The results of beam optimization are presented. Parameters achieved are as follows: beam current  24A, power  480kW, duration  20 ms. Local data acquisition was arranged using 16-channel PC-based ADC, which was protected by copper shield and kept at the injector potential level (~20kV). ADC was connected to TUMAN-3M acquisition system through optical fiber. This approach proved to provide sufficient acquisition stability.

### References

- Vorobjev G.М. et al. “Plasma Physics”, v 9, 1983, p. 105.

- Askinazi L.G., et al, “Plasma Devices and Operations”, v 11, 2003, pp. 211-218.

- Merezhkin А.А., Mukhovatov V.S. “Letters in JETP”, v 33, 1981, p. 463.

- Barsukov A.G. et al, Proc IX IAEA Conf. On Plasma Phys. and Control Nucl. Fusion Res, IAEA, 1982, v 1, p. 83.