Numerical simulation of rod-pinch diode

I.V. Glazyrin, A.V. Karpeev, O.G. Kotova

Russian Federal Nuclear Center – VNII Technical Physics named after acad. E.I. Zababakhin, Snezhinsk, Russia, e-mail: a.v.karpeev@vniitf.ru

2D numerical PIC program calculating self-consistent motion of relativistic electron and ion beams in electro-magnetic fields is used to analysis of electron beam focusing in rod-pinch diode. Current through the anode surface is accounted during calculations of magnetic field.

Two configurations of rod-pinch diode is analyzed: with small aspect ratio [1] and with large one. Work of the latter configuration is studied on IGUR accelerator (RFNC-VNIITF) [2]. Calculation results are close to the experiment data for the first diode. For IGUR rod-pinch diode motion of anode plasma must be taken into account. Calculation results of different cathode-anode gaps and velocities of plasma expansion are presented. Focusing conditions of electron beam were found.

References

- Swanekamp S.B., Allen R.J., Commissio R.J. et al. Angular dose variation from 4- to 6-MV rod-pinch diode experiments on the ASTERIX pulsed power generator // Proc. 14th IEEE Inter. Pulsed Power Conf., PPC-2003, Dallas, Texas, USA, June 15-18, 2003, pp. 483-486

- Kormilitsyn A.I., Kononenko V.Yu., Vedernikov A.I., Kasianov N.Yu., Koloskov E.N., Pokrovskii A.S. Research of rod-pinch operation opportunity of the 4 MV on IGUR accelerators, Thesis of International Conference Zababakhin Scientific Talks, Snezhinsk, Russia, September 5-10, 2005, p.109.