x-ray creation in ecr plasma source cera-r

Andreev V.V., Balmashnov A.A., Kalashnikov A.V., Umnov A.M.

PFU, Moscow, Russia, e-mail: abalmashnov@sci.pfu.edu.ru

ECR plasma source CERA-RX can create in radial direction X-ray azimuth symmetric flow with quant energy (19-22) keV and density up 40 mkR/c.cm2 at the distance 40 cm from the axis of the source. Microwave power (f=2,47 GGz) was not higher than 80 W.

CERA-X consists from the coaxial resonant cavity with different cross section contained by constant magnets. Diameter of the source is 9 cm, length – 14 cm. Working gas pressure (Ar, N2, H2) was (2-10) 10-5 Torr.

It was established:

  - No dependence of quant energy on type and pressure of working gas and microwave power (up 80 W).

  - Dependence X-ray intensity on type and pressure of working gas and microwave power.

  - Dependence of quant energy on magnetic field profile.

We suppose that this type of source can be used to create X-ray with quant energy up some hundred keV.

At present time the compact X-ray ECR plasma source with quant energy (20-40) keV has been developed.