TWO-STAGE GENERATION OF TERAHERTZ RADIATION PULSES ON BASIS OF 2D BRAGG PLANAR FEM BY INTERCAVITY STIMULATED SCATTERING

A.V. Arzhannikov, N.S. Ginzburg*, P.V. Kalinin, S.A. Kuznetsov, N.Yu. Peskov*, R.M. Rozental*, A.S. Sergeev*, S.L. Sinitsky, V.D. Stepanov, M. Thumm**, V.Yu. Zaslavsky*, I.V. Zotova*

Budker Institute of Nuclear Physics, Novosibirsk, Russia, e-mail: arzhannikov@inp.nsk.su
*Institute of Applied Physics, N-Novgorod, Russia
**Institute fur Hochsleistungsimpuls- und Mikrowellentechnik, Karlsruhe, Germany

In the paper we propose a novel variety of a two-stage scheme of generation of short wavelength radiation based on stimulated inter-cavity scattering. At the first stage a sheet electron beam drives a 2D Bragg free electron maser (FEM) of planar geometry to generate 4-mm pump wave [1]. At the second stage this wave undergoes stimulated scattering at the supplementary electron beam to produce submilimeter radiation. A key feature of a proposed scheme is to use two parallel sheet beams from single diode with two cathodes at a few kiloampers currents that transported in two parallel channels, which are connected by special waveguide. Such a scheme allows us to use high power 4-mm radiation produced in the first generation channel by the sheet beam moving in an undulator quasi-static magnetic field, as an EM-undulator of the secondary stage FEL operating in submillimeter wave range.

Paper describes main features of a project of an experiment at the ELMI- device on two-stage generation of submillimeter radiation

References

- A.V.Arzhannikov, V.B. Bobylev, N.S.Ginzburg et al. “One-channel and multi-channel  planar masers”, Izvestiya Vuzov, Radiofizika, 46 (2003), 10, p. 907-913 (in Russian)