Current ramp scenarios calculation for Component Test Facility with DINA code.

A. Dnestrovskij, V. Lukash, R. Khayrutdinov*

RRC Kurchatov Institute, Moscow Russia
*TRINITI, Troitsk, Russia

The free boundary equilibrium time evolution code DINA [1] is implemented for calculations of the current ramp up scenarios in Component Test Facility (CTF) machine [2,3] based on the concept of low aspect ratio tokamaks. One of the problems of this concept is the absence of the central solenoid which used in tokamaks for the discharge breakdown and current ramp up. This problem is connected with high thermal and neutron load as well as the little room in the torus center for the solenoid. The current ramp up and subsequent support at the steady phase operation in CTF is supported by the current driven by the Neutral Beam (NB), bootstrap current and the current produced due to the change of the poloidal field from the control coils (BV–effect). The problems of seeded current formation are discussed. The self-consistent scenario for current ramp up for CTF project is worked out. The steady state regime parameters are analyzed.. NB absorption and current drive calculations are carried out by the code implemented in DINA code and tested on different devices [4].

Calculations with NUBEAM Monte Carlo code [5] are performed and presented in the report

References

- R.R. Khayrutdinov and V.E. Lukash, Studies of Plasma Equilibrium and Transport in a Tokamak Fusion Device with the Inverse-Variable Technique,  Journal of Comp. Physics, 107 (2), 106  (1993)

- H.R. Wilson, G.M. Voss, R.J. Akers, et al. A Steady State Spherical Tokamak for Components Testing, in Proceedings of the 20th IAEA Fusion Energy Conference, Invited Paper FT/3-1Ra;

- A.W.Morris, R.J.Akers, G.F.Counsell, et al. Spherical Tokamaks: Present Status and Role in the Development of Fusion Power, Fusion Engineering and Design 74, 67 (2005)

- E. A. Azizov, Yu.A.Arefiev, G.G.Gladush, et al. The Concept of the Volumetric Neutron Source on Basic of the JUST-T Tokamak for Minor Actinides Transmutation, Plasma Devices and Operations, 11, No.4, 279 (2003)

- D.McCune NTCC Modules Library: NUBEAM http://w3.pppl.gov/~pshare/help/nubeam.htm