А method for assessment of neutral TRITIUM-TO-DEUTERIUM ratio MEASUREMENT accuracy in THE SOL from high resolution H-alpha spectroscopy in ITER

V.S. Neverov, A.B. Kukushkin

TPI NRC “Kurchatov Institute”, Moscow, Russia, vs-never@mail.ru

The inverse problem of neutral atom T/D ratio recovery in the SOL from high resolution H-alpha spectroscopy measurements is examined for ITER conditions. A prototype of the numerical code to solve this problem is elaborated, which allows using as a constituent blocks both the already existing and extendable databases (a, b) and the modules being developed on the base of new models by other authors (c, d):

- numerical data for temperatures and densities of the major components of a deuterium plasma, calculated by the SOLPS4.3 (B2-EIRENE) transport code [1]-[3],

- numerical data [4] for space-velocity distribution function of neutral deuterium atoms, calculated by the EIRENE code [2] applied on the background of the above data,

- semi-analytic 1D model for kinetics of neutral atoms of all hydrogen isotopes in the SOL [5],

- semi-analytic model for the spectra of the light emitted in Balmer lines in divertor and reaching the spectrometers after diffusive or multiple mirror reflections from the all-metal first wall [6].

Test calculations to estimate the accuracy of measurements are carried out. A degree of suppression of the scattered Balmer light from divertor, which is needed to obey the ITER Measurement Requirements, is evaluated.

References.

- Braams B.J. PhD thesis. Utrecht: Rijksuniversitet, 1986.

- Reiter D., Baelmans M., Boerner P. Fusion Sci. Tech., 2005, 47, 172 (www.eirene.de).

- Kukushkin A.S., Pacher H.D., Loarte A., Kotov V., et al. Nucl. Fusion, 2009, 49, 075008.

- Kotov V. Private communication, 2011.

- Kadomtsev M.B., Lisitsa V.S., Shurigin V.A. Private communication, 2011.

- Kukushkin A.B., Levashova M.G. Private communication, 2011.