theoretical problems of high resolution H- spectroscopy measurements in ITER

A.B. Kukushkin, V.S. Lisitsa, M.B. Kadomtsev, M.G. Levashova, V.S. Neverov, V.A. Shurygin, *V. Kotov, **A.S. Kukushkin, **S. Lisgo, A.G. Alekseev, A.V. Gorshkov, D.K. Vukolov, K.Yu. Vukolov, **E. Veshchev

TPI NRC “Kurchatov Institute”, Moscow, Russia, kuka@nfi.kiae.ru
*Forschungszentrum Jülich GmbH, IEK-4-Plasma Physics, Jülich, Germany
**ITER Organization, Cadarache, France

A survey of theoretical problems of high resolution H-alpha spectroscopy measurements in ITER, which have been under consideration at the stage of the H-alpha Spectroscopy Diagnostic Conceptual Design Review, is given. These include:

- comparative analysis of predictive numerical modeling of 2D spatial distributions of plasma parameters (densities and temperatures) in the divertor and SOL at a steady-state stage of discharge in ITER with the SOLPS4.3 (B2-EIRENE) [1-3] code,

- semi-analytic, 1D model of neutral atoms velocity distribution function (VDF) in the SOL and its comparison with the EIRENE stand-alone simulations of neutral deuterium VDF, applied on the plasma background calculated by the B2-EIRENE (SOLPS4.3) code,

- semi-analytic model for the spectra of the light emitted in the Balmer lines in divertor and taken by spectrometers after diffusive or multiple mirror reflections from all-metal first wall (divertor stray light (DSL) problem),

- formulation of an inverse problem for assessment of tritium-to-deuterium ratio and total neutral density recovery in the SOL from high resolution H-alpha spectroscopy measurements,

- assessment of the above measurements’ accuracy under condition of a substantial dominance of DSL spectral intensity over that for the Balmer-alpha emission from the SOL for a number of lines of sight in ITER.

Plans for future work, including a test of the elaborated approach in the currently running machines with all-metal first wall, are presented.

References.

- Kukushkin A.S., Pacher H.D., Loarte A., Kotov V., et al. Nucl. Fusion, 2009, 49, 075008.

- Braams B.J. PhD thesis. Utrecht: Rijksuniversitet, 1986.

- Reiter D., Baelmans M., Boerner P. Fusion Sci. Tech., 2005, 47, 172 (www.eirene.de).