Changes in Short-Wave Turbulence in Plasma of the L-2M Stellarator under Different ECRH Conditions

G.M. Batanov, L.V. Kolik, A.E. Petrov, A.A. Phenichnikov, K.A. Sarksyan, N.N. Skvortsova, N.K. Kharchev

A.M. Prokhorov General Physics Institute, Russian Academy of Sciences

In an attempt to explain anomalous transport in toroidal confinement systems, recent attention has been focused on short-wave turbulence associated with ETG (Electron Temperature Gradient driven modes) instability (see, e.g., [1-3]). Short-wave turbulence (kтете/е~0.1-0.2) was observed in the L-2M and LHD stellarator by using Bragg scattering of gyrotron radiation at the operating frequency and its second harmonics [4, 5]. The excitation of short-wave turbulence (kте/е~0.3-0.4) in plasma of the FT-2 tokamak was studied by using enhanced scattering diagnostics [6].

The report presents results of studies of characteristics of short-wave turbulence measured by Bragg scattering of gyrotron radiation at the second harmonic of operating frequency (k40 cm-1, kтe/е0.2) and also long-wave plasma fluctuations measured by small-angle scattering of gyrotron radiation at the operating frequency (k1-1.5 cm-1, kтi/i0.1).  Measurements were carried out at ECRH powers of 100 and 200 kW. A sector limiter was used in some discharges to restrict the plasma radius.

The Fourier spectra of scattered radiation in both cases of Bragg and small-angle scattering are continuous spectra with exponentially decreasing intensity in the frequency range from 5 kHz to 1 MHz and with feebly marked wide bands. As the ECRH power was doubled, the noise spectral density increased by a factor of 1.5-2 throughout the measured spectra. Under these heating conditions, the electron temperature in the Bragg scattering region increased by a factor of 1.5. When the sector limiter was introduced into the plasma to a depth of 2 cm, the electron temperature was lowered by a factor of about 1.5; at the same time, the noise spectral density increased by a factor of 1.5-2 in the frequency range 10-150 kHz for both k40 cm-1 and k1-1.5 cm-1. In the Bragg scattering spectra (k40 cm-1) measured in the region r/а = 0.3-0.4, situated nearer to the plasma axis, the intensities of particular spectral bands seen against the background of continuous spectrum were higher in comparison with the spectra measured in the region r/а = 0.5-0.6. The small-angle scattering (k1-1.5 cm-1) spectra were similar to the Bragg scattering spectra from the region r/а = 0.3-0.4. Estimates show that the conditions for an onset of ETG instability are satisfied in L-2M plasma at r/a 0.3-0.6. It may be assumed that a correlation between values of the electron temperature and spectral intensity of short-wave turbulence observed under different discharge conditions (ECRH power, plasma--wall interaction) is some evidence of interrelation between transport processes and short-wave turbulence.

References

- Jenko F., Dorland W., Hammet G.W. Phys. Plasmas. 2001.8. 4096

- Li J., Kishimoto Y. Phys. Plasmas. 2004. 11. 1493.

- Ronch C.M. et al. Plasma Phys. Control. Fusion. 2005. 47. B323.

- G.M. Batanov, L.V. Kolik, А.Е. Petrov et al. Plasma Physics Reports. 2003. 29. 380.

- G.M. Batanov, L.V. Kolik, M.I. Petelin et al. Plasma Physics Reports. 2003. 29. 1019.

- Gurchenko A.D., Gusakov E.Z., Altukhov A.B. et al. Nucl. Fusion. 47. 245.