Special Energy Absorption Properties of a Target for ISKRA-6

Atamanenko V.D., Dolgoleva G.V., Shcherbakov V.A.

RFNC-VNIIEF, Sarov, Russia

Special energy absorption properties of an indirect-drive cryogenic target for the ISKRA-6 laser facility observed in simulations using the SND code are discussed.

1	The amount of energy absorbed for the Cu content in the Be ablator of  αCu=2% and αCu=8%is approximately the same.

2	Nearly all the ablator is heated, which may lead to the heating of the frozen DT fuel layer and thus adversely affect target performance.

3	Radiation flux to the target in the spectral multigroup approximation for some groups is directed outwards.

4	For αCu=8%, in a certain region of the ablator, the material is heated above the radiation temperature.