GENERALIZATION OF TRUBNIKOV FORMULA FOR Electron Cyclotron TOTAL POWER LOSS In Tokamak-Reactors

A.B. Kukushkin,1 P.V. Minashin1,2

1NFI RRC "Kurchatov Institute", Moscow, Russia, kuka@nfi.kiae.ru
2National Research Nuclear University «MEPHI», Moscow, Russia

Electron cyclotron radiation (ECR) can significantly influence the local energy balance in the central part of the plasma column in the steady-state regimes of tokamak-reactor operation. The necessity to model such regimes with the fast transport codes requires parameterization of the ECR total (i.e. volume-integrated) power loss, .

Here we propose a new formula for , which fits the numerical results from the code CYNEQ [1,2] for a wide range of Te profiles (for profiles with a pedestal, with an accuracy less than ~25%) and gives a simplest extension of the Trubnikov’s formula [3], suggested for homogeneous profiles of electron density, ne, and temperature, Te, to the case of inhomogeneous ne and Te profiles, expected for fusion reactor-grade tokamaks, including ITER and DEMO:

(1)

where the effective temperature  differs from the volume-averaged temperature, while effective density coincides with the volume-averaged density,

,          .	(2)

In Eq. (1), the density is in 1020 m-3, temperature, in keV;  is plasma volume; major, R, and effective minor radius, allowing for elongation k, aeff=a(k)1/2, are in meters; magnetic field on axis, B0, in Tesla.

As distinct from the most successful analytic formulae for inhomogeneous plasmas in tokamaks - namely, formulae in [4,5], formula (1) does not assume analytic representation of Te and ne profiles or a fitting of actual Te and ne profiles with the profiles from the class considered in [4,5].

References

- Kukushkin A.B., Proc. 14th IAEA Conf. on Plasma Phys. and Contr. Fusion, Wuerzburg, Germany, 1992, v.2, pp. 35-45.

- Cherepanov K.V., Kukushkin A.B., Proc. 20th IAEA Fusion Energy Conf., Vilamoura, Portugal, 2004, TH/P6-56.

- Trubnikov B.A., in Reviews of Plasma Physics, vol. 7 (M.A. Leontovich, Ed.), New-York: Consultants Bureau, 1979, p. 345.

- Albajar F., Johner J., Granata G. Nuclear Fusion, 2001, 41, No. 6, 665-678.

- Fidone I., Meyer R.L., Giruzzi G., Granata G. Phys. Fluids B, 1992, 4, 4051.