Ion acceleration by a moving virtual cathode on decay of a compressed electron beam

A.A. Grishkov, S.Ya. Belomyttsev, V.V. Ryzhov, V.A. Shkyaev

Institute of High Current Electronics SB RAS, Tomsk, Russia, e-mail: shklyaev@to.hcei.tsc.ru

Theoretical and numerical studies of the collective acceleration of positively charged ions by a moving virtual cathode in a cylindrical two-section transport system were conducted. It is shown that on decay of a compressed electron beam, the mechanism of ion acceleration is more efficient than that during the formation of a compressed beam, where the virtual cathode shits toward the injection plane.

The collective ion acceleration was simulated using the PIC code KARAT [1]. The calculations were made for р, He+, C+, Fe+ ions. The residual gas pressure was from 10-3 to 10-5 torr. The injected electron energy was varied between 150 keV and 1.5 MV. For the SINUS-7 accelerator parameters under study, the output ion energy was several times greater than the injected electron energy.

References.

- V.P. Tarakanov, User’s Manual for Code Karat, BRA,  Springfield, 1992.