On the role of electron cyclotron radiation losses in tokamak ITER steady-state REGIME

A.B. Kukushkin, P.V. Minashin, *A.R. Polevoi

TPI RRC“Kurchatov Institute”, Moscow 123182, Russia, e-mail: kuka@nfi.kiae.ru
*ITER Organization, CS 90 046, 13067 St Paul lez Durance Cedex, France, 
     e-mail: Alexei.Polevoi@iter.org

Electron cyclotron radiation (ECR), PEC, can significantly contribute to the local electron power balance in central part of plasma for high temperatures, Те, expected in steady-state regimes of ITER operation (see, e.g., ). Here we analyze the role of ECR power losses for parameters needed for the steady-state operation of ITER [2] (regime with complete replacement of inductive current), predicted with the transport code ASTRA [3]. To this end, the simulator (fast routine) of the CYNEQ code [4-5] for calculation of ECR power density profile, PEC(ρ), is incorporated into ASTRA to calculate the local electron energy balance self-consistently with 1D transport and 2D equilibrium (1.5D approximation). We use the CYNEQ version [5,6] which extends the version [4] to a full account of the inhomogeneity of the magnetic field in the plasma column in axisymmetric approximation.

It is shown that when central temperature increases to Te(0) ~ 30 keV the local EC power loss becomes a substantial part of the heating from fusion alphas and is close to the total auxiliary heating (with neutral beam injection), PEC(0)  0.3Pα(0)  Paux(0). For Te > 35 keV the ECR losses have a positive impact on stabilization of fusion burning (d(Pα - PEC)/dT < 0). This shows the necessity of using CYNEQ [5,6] for increasing the accuracy of self-consistent simulations of plasma parameters in the fusion reactor-grade tokamaks.

References.

- Albajar F., et al., Nuclear Fusion, 2005, 45, 642-648.

- Polevoi A.R., et al. Proc. 37th EPS Conf. on Contr. Fusion and Plasma Phys., Dublin, Ireland, 2010, P2.187.

- Pereverzev G.V., Yushmanov P.N., Report IPP 5/98, 2002.

- Kukushkin A.B., Proc. 14th IAEA Conf. on Plasma Phys. and Contr. Fusion, Wuerzburg, 1992, 2, 35-45;Cherepanov K.V., Kukushkin A.B., Proc. 20th IAEA Fusion Energy Conf., Vilamoura, 2004, P6-56.

- Kukushkin A.B., Minashin P.V., EPS-2009. http://epsppd.epfl.ch/Sofia/pdf/P4_136.pdf

- Kukushkin A.B., Minashin P.V., Polevoi A.R., Proc. 23rd IAEA Fusion Energy Conference, Korea, October 2010.
http://www-pub.iaea.org/mtcd/meetings/PDFplus/2010/cn180/cn180_papers/itr_p1-34.pdf