Port plug test facility for vacuum, thermal and functional tests-new contribution of Russian Federation to ITER creation

Romannikov A., *Skorodumov B., Krasilnikov A., *Besrukov K., **Levesy B., **Bruno L., *Kotusov S., *Kuzmenko I., *Rumyantsev Yu. and JSC “Cryogenmash” team

1 ITER Agency, Russian Research Center «Kurchatov  Institute», pl. Kurchatova I., 1,
     Moscow 123182, Russia  a.romannikov@iterrf.ru
*JSC “Cryogenmash”, 143907 Moscow reg., Balashikha
**ITER Organization, Route de Vinon sur Verdon 13115 St Paul Lez Durance – France

The port plug test facilities are used for:

- Pre-acceptance testing

  - Initial vacuum test of fully assembled plugs

  - 3 thermal cycles under vacuum

  - Helium leak check

  - Functional tests (shutters, mirrors, RF conditioning, etc.)

  - Ideally at DA sites, so that repairs can be made without shipping delays

  - Performed after shipment to ITER

  - For diagnostic plugs, after attachment of blanket shield modules at ITER site

  - 1 thermal cycle followed by helium leak check.

  - For IC antenna: integrated tests of antenna, transmission lines, generators and control system

- Tests in Hot cell

  - Similar to pre-installation test and limited number of functional tests

The following upper and equatorial port plugs have to be tested: IC system, EC system, Diagnostics, TBM (Test Blanket Module). In total, 32 port plugs have to be tested.