Modeling of the turbulence wave number reconstruction using radial correlation reflectometry data with applications to FT-2, JET, Tore Supra like cases

Gusakov E.Z., Kosolapova N.V., *Heuraux S.

Ioffe Institute, St.Petersburg, RF, n.kosolapova@mail.ioffe.ru
*LPMIA, UMR-CNRS 7040, UHP, Nancy, France

Radial correlation reflectometry (RCR) is widely used for hot plasma density diagnostics in tokamaks. It uses simultaneously different frequencies for probing and the coherence decay of two signals with growing difference of probing frequencies is studied by such a diagnostics [1]. The aim of RCR is determining plasma density micro turbulence parameters in particular wave number spectrum or space cross correlation function (CCF).

The data interpretation in this technique is based on the hypothesis of high measurements localization. In essence, it is similar to that used in probe correlation method where the signal CCF is identified with the turbulence CCF. Unfortunately according to strict theoretical analysis, both numerical and analytical [2] and [3], this assumption is incorrect; nevertheless the one dimensional analytical theory shows in the present paper it is possible to reconstruct the turbulence spectra using the RCR experimental data.

In the paper the results of numerical modeling of micro turbulence wave number reconstruction using RCR for FT-2, Tore Supra and JET like cases are presented. It is shown that turbulence CCF is reconstructed more closely than spectra. The technique for internal transport barriers (ITB) detecting and measuring is proposed as well.

The work is subsidized by RFBR 09-02-00453, 07-02-92162-CNRS grants and RFBR – NWO Centre of Excellence on Fusion Physics and Technology (grant 047.018.002) and Saint-Petersburg Government.

References

- N. Bretz, 1992 Physical Fluids B4(8) 2414

- H. Hutchinson, 1992 PPCF 34 1225

- G. Leclert, S. Heraux, E.Z. Gusakov, A.Yu. Popov, I. Boucher, L. Vermate, 2006 PPCF 48 1389

- E. Gusakov, A. Popov, 2002 PPCF 44 2327