- OBSERVATION OF A SHOCK WAVE AND  AN IONIZATION FRONT PROPAGATION  IN A MICROWAVE BEAMING THRUSTER

Y. Oda, K. Komurasaki, K. Takahashi*, A. Kasugai* and K. Sakamoto*

The University of Tokyo, Kashiwa, Japan,
*Japan Atomic Energy Agency, Naka, Japan

Microwave beamed energy propulsion is an application of atmospheric discharge in a high power microwave beam. When a high power pulsed microwave beam is provided into a focusing reflector, atmospheric discharge arises in the vicinity of the focal point. The induced plasma absorbs the following part of the microwave pulse and expands outwards while generating a shock wave. The shock wave drives impulsive force to the reflector. Then repetitive microwave pulses produce propulsive thrust. Because propulsive energy is provided by a microwave beam transmitted from outside, the vehicle is not necessary to load an energy source by itself. We had conducted a flight experiment using a 1MW-class 170GHz-gyrotron. The measured momentum coupling coefficient Cm, defined as a ratio of propulsive impulse to input power, was over 400N/MW with a thruster model with cylindrical tube. [1]

In a thruster with a cylindrical tube, an ionization front propagates in the tube absorbing microwave power in a supersonic speed accompanying a shock wave. Therefore, the propagation model of a shock wave in a detonation tube is expected useful. In this study, a pressure history in the thruster was measured using pressure gauges and compared to the analytical model. At the same time, propagation velocity of the ionization front was measured using a high-speed framing camera.

As a result, the propagation velocities of the shock wave and ionization front were found identical. Measured pressure at the thruster wall showed a similar history to that from the detonation tube model.

- Nakagawa, T., Mihara, Y., Komurasaki, K., Takahashi, K., Sakamoto, K., and Imai, T., Journal of Spacecraft and Rockets, 2004, 41, 151