- MICROWAVE ULTRAVIOLET LAMP

E.M. Barkhudarov, M.A. Misakyan, I.A. Kossyi, V.A. Kiselev*, I.M. Taktakishvili, A.V. Tulupov*

General Physics Institute of Russian Academy of Sciences, Moscow, Russia
*“SOLITON-NTT” Co. Ltd., Moscow, Russia

Microwave gas-discharge ultraviolet lamp (MW-UV-lamp) has been built up in the Plasma Physics Division of General Physics Institute of Russian Academy of Sciences (GPI).

Unusual method of microwave discharge excitation in a lamp bulb has been developed. This method based on a principle of “truncated electrode of coaxial” has been formulated previously (see [1]) allows excite microwave discharge in gases and gaseous mixtures over a wide range of pressures at a relatively low microwave power. Ordinary magnetrons used in modern domestic microwave ovens (wavelength near 12 cm, power about 600-1000W) have been utilized in a MW-UV-lamp that is now in operation. Principles of microwave energy input in an “open” lamp as well as a lamp introduced in water have been elaborated.

The main preferences of this construction are as following:

- Absence of electrodes that noticeably increases of UV-lamp life time;

- Simplicity and cheapness of water disinfection system based on the MW-UV-lamp application.

Preliminary microbiological researches have shown that the productivity of lamp regarding to the disinfection of microbially contaminated water (reduction of microbial content in  103 less than the initial one) is as low as:

  10-3 kW hour/liter.

For experimental investigation of spectral characteristics of MW-UV-lamp optical and chemical (actinometric) methods developed in GPI (see [2,3]) have been targeted for application. With help of these methods optimal buffer gas pressure for biologically active UV radiation has been determined.

References

- S.I. Gritsinin, I.A. Kossyi et al,  Proceedings of 14th International Symposium on Plasma Chemistry, Prague, 1999, v. II, 675.

- Yu.B. Bark, E.M. Barkhudarov, I.A. Kossyi et al., J. Phys. D: Appl. Phys., v. 33, 2000, 859.

- A.M. Anpilov, E.M. Barkhudarov, Yu.B. Bark et al,  J. Phys. D: Appl. Phys., v. 34, 2001,  993.